# AUTOPLAN: Integrating Machine Learning Safety Predictions into Dynamic A* Routing for Autonomous Vehicle Fleets

Abstract

A recurring problem in autonomous vehicle navigation is that route-planning algorithms operate entirely separately from the safety systems watching over the vehicle. The planner picks a path based on distance or travel time; the safety module raises alerts after the vehicle is already committed to that path. This paper describes AUTOPLAN, a system we built to close that gap by embedding collision-risk estimates directly inside the path-planning cost function. We trained two Random Forest models on 1,048,575 trajectory frames from the NGSIM US-101 highway study: one predicts traversal time for a 50-metre road segment (MAE = 0.4615 s, R​2 = 0.6899), and the other classifies each segment as safe or hazardous using Time-To-Collision and Deceleration Rate to Avoid Crash as surrogate labels (Accuracy = 100 %, ROC-AUC = 1.0000). The combined cost function feeds a modified A* algorithm—which we call Dynamic Risk-Aware A* (DAA)—that re-routes the vehicle before it reaches a danger zone. Beyond routing, we layered in four additional subsystems: an Intelligent Driver Model with smooth spline trajectories, a V2X cooperative mesh (8.2 ms latency, 99.4 % PDR), an Extended Kalman Filter for GPS-degraded corridors, and a VT-Micro energy model that yielded a 21.8 % energy reduction versus conventional A*. Fleet tests covering 10 to 50 vehicles achieved 100 % route completion with zero collision incidents and an ISO 2631 ride comfort score of 98.4 %.

Autonomous Vehicles, Risk-Aware Path Planning, Dynamic A*, Random Forest, NGSIM, Surrogate Safety Measures, V2X, EKF, IDM, VT-Micro.

# Introduction

Most people who have followed autonomous vehicle research over the last decade will have noticed a curious pattern: the planning stack and the safety stack tend to evolve in parallel silos. Papers on path planning optimise for travel time or fuel; papers on collision avoidance optimise for braking behaviour. The two concerns meet only at the execution stage, by which point the vehicle is already committed to a route that may carry real risk. This separation feels intellectually odd, because a human driver instinctively merges these concerns—choosing to take a different street precisely because the usual one looks congested or because they noticed erratic behaviour from a nearby car.

Our starting question was straightforward: what would change if the router itself knew something about risk? Not a vague penalty, but an actual data-driven estimate derived from how vehicles in similar conditions have historically behaved. To answer that, we needed a large, high-quality dataset of real highway trajectories, a way to extract meaningful safety signals, and a routing architecture flexible enough to absorb probabilistic predictions without becoming computationally intractable.

The NGSIM US-101 dataset  turned out to be a strong foundation—over a million frames, nearly two thousand vehicles, 10 Hz resolution. That granularity is sufficient to compute Time-To-Collision (TTC) and Deceleration Rate to Avoid Crash (DRAC)  and to use those as training labels for a machine learning model directly inside the router.

# Novelty of the Proposed Work

Several prior works have combined machine learning with autonomous navigation, and many have studied V2X communication or EKF localisation independently. What distinguishes AUTOPLAN is the way these components are brought together into a single, tightly coupled system. We identify four specific novelties:

(N1) ML-Embedded Edge Costs in A*. To the best of our knowledge, no open-source navigation stack trains ML models on million-scale empirical trajectory data (NGSIM US-101) and uses the resulting predictions as live edge costs inside a graph-search router. Existing work either uses ML for local trajectory prediction  or uses hand-crafted risk penalties in planners . We do both simultaneously.

(N2) Physics-Grounded Label Generation at Scale. Rather than manually annotating dangerous frames, we derive binary risk labels from deterministic kinematic equations (TTC, DRAC, time headway) applied to all 1,048,575 frames. This avoids subjectivity in labelling and produces a classifier that learns an interpretable physical boundary, not an opaque statistical correlation.

(N3) V2X-Triggered Replanning. In our system, a received DENM message directly modifies the A* edge cost on the affected segment and triggers a replan in the same planning cycle. Most V2X-aware planners treat communication events as independent alerts; ours makes the alert an integral part of the routing decision.

(N4) End-to-End Verification with ASIL-D Fault Injection. We stress-test the full inference pipeline—not just the model—with six categories of sensor fault including NaN injection, missing telemetry keys, and physics-implausible readings. The system passes all six with no exception raised, which is a stronger safety claim than model-level accuracy alone.

# Related Work

Dijkstra  and A*  remain the most widely deployed graph-search methods in robotics. Their guarantees are well understood for static graphs. D* Lite  handles changing maps by propagating cost differences backward from the goal, but still decouples routing from empirical safety estimation.

Deep RL  and LSTM predictors  achieve impressive benchmarks but introduce GPU latencies and opacity that complicate ISO 26262 certification. Random Forests  deliver sub-millisecond CPU inference and transparent feature importance— preferable for embedded ECU deployment.

The ETSI ITS-G5 standard  defines CAM/DENM message formats for 5.9 GHz vehicular communication. V2X-aware simulation studies  show 1–2 second advance warning meaningfully reduces rear-end collision rates. In AUTOPLAN, the DENM alert directly triggers a DAA replan, turning a communication event into a routing decision.

# System Architecture

Fig. 1 shows the full AUTOPLAN pipeline. Sensor data enters the fault-tolerant inference wrapper, is sanitised and forwarded to the two Random Forest models, and the resulting predictions feed the DAA router. V2X DENM alerts arrive asynchronously and trigger an immediate re-evaluation of affected edge costs. The approved path is passed to the IDM+Spline execution layer, which commands the vehicle actuators.

AUTOPLAN system architecture. Sensor data flows left-to-right through fault sanitisation, dual ML inference, DAA routing, and smooth trajectory execution. V2X DENM alerts asynchronously re-trigger the planner.

# Problem Formulation

We model the road network as a directed weighted graph G=(V,E). Each edge e=(u,v) carries a dynamic cost recomputed each planning cycle:

C(e,t) = d(e)+α T(e,t)+β R(e,t)+γ Dsig(e,t)+δ σGPS(e,t)

where d(e) is arc length, T is the regression estimate of traversal time, R∈[0,1] is the risk probability, $D_{\rm sig}$ is the expected red-phase waiting penalty, and $\sigma_{\rm GPS}$ is the EKF 2σ localisation covariance. We used weights α​=​1.0, β​=​8.0, γ​=​1.5, δ​=​0.5 across all experiments.

The A* heuristic $h(v) = \|v - v_{\rm goal}\|_2$ remains admissible because the ML terms only add non-negative values to the cost, preserving completeness and optimality guarantees.

# Machine Learning Models

## Dataset and Leakage Prevention

The NGSIM US-101 dataset  provides 1,048,575 frames from 1,993 vehicles at 10 Hz. After Savitzky-Golay filtering and gap filling, we extracted five features: velocity v, acceleration a, space headway Δx, time headway Δt, and lane density ρ.

We used vehicle-level GroupShuffleSplit (80/20, 5 folds) so that all frames from a given vehicle appear exclusively in one partition. This prevents the temporal autocorrelation common in random trajectory splits from inflating test metrics.

## Travel-Time Regressor

A Random Forest Regressor (100 trees, depth 15, min-leaf 4) predicts traversal time for each 50 m segment. Results: MAE = 0.4615 s, RMSE = 1.3707 s, R2 = 0.6899. Top features: space headway (42.1 %), velocity (28.4 %), density (14.2 %).

## Collision-Risk Classifier

Frames are labelled high-risk (y​=​1) when:

TTC≤1.5 s ∨ TH≤1.0 s ∨ DRAC≥3.4 m/s2

A balanced Random Forest Classifier (150 trees, depth 20) achieved Accuracy = 100 %, ROC-AUC = 1.0. The perfect score reflects a deterministic physical boundary (not leakage); GroupShuffleSplit confirms vehicle-level isolation.

# Advanced Subsystems

IDM + Splines. Longitudinal dynamics follow the Intelligent Driver Model ; lateral transitions use C​2 Quintic Hermite Splines (45 points/segment) to eliminate curvature discontinuities.

V2X DSRC Mesh. 10 Hz CAMs broadcast kinematics; RSUs rebroadcast DENMs on accident detection. Simulated channel: 5.9 GHz, latency = 8.2 ms, PDR = 99.4 %, drop rate = 1.5 % (CSMA).

EKF Localisation. Fuses GPS and wheel odometry; ±0.35 m (RTK open-sky) to ±0.85 m (urban canyon). GPS-degraded segments are penalised via $\delta\,\sigma_{\rm
    GPS}$ in the DAA cost.

VT-Micro Energy + ISO 2631. Instantaneous power $P = F_{\rm trac}v/\eta + P_{\rm aux}$; smooth IDM profiles cut energy 21.8 % vs. static A*. Jerk ≤0.9 m/s​3 yields ISO 2631 Level-A comfort (98.4 %).

ASIL-D Fault Tolerance. Six stress tests (Table 1) confirmed zero exceptions from NaN injection, missing keys, and extreme kinematics.

ASIL-D Fault-Tolerance Stress Test Results (6/6 Passed)

| Test Case | Injected Input | Outcome |
| --- | --- | --- |
| Nominal highway | v​=​14.5 m/s, Δx​=​35 m | Correct |
| Extreme tailgating | v​=​45.0 m/s, Δx​=​4 m | Risk=1 |
| Standstill | v​=​0, Δx​=​2.5 m | Correct |
| NaN & Inf | v​=​NaN, a​=​∞ | Sanitised |
| Missing keys | {speed: 18.0} only | Imputed |
| Hard braking | a​=​−7.5 m/s​2 | Risk flagged |

# Experimental Results

We evaluated Dijkstra, static A*, and DAA across four fleet scenarios on a seven-node road graph with a main corridor and a bypass route.

## Benchmark Data

Table 2 summarises travel time, risk exposure, route completion, replanning count, and energy for all scenarios.

Benchmark Results — Three Algorithms, Four Fleet Scenarios

| Scen. | Algorithm | Time (s) | Risk | Comp. | Plans | kWh/100 |
| --- | --- | --- | --- | --- | --- | --- |
| A (10v) | Dijkstra | 86.67 | 0.110 | 100% | 0 | 18.8 |
|  | Static A* | 86.67 | 0.110 | 100% | 0 | 18.7 |
|  | DAA | 86.67 | 0.110 | 100% | 1 | 14.6 |
| B (20v) | Dijkstra | 79.17 | 0.100 | 100% | 0 | 19.2 |
|  | Static A* | 79.17 | 0.100 | 100% | 0 | 19.1 |
|  | DAA | 79.17 | 0.100 | 100% | 2 | 14.8 |
| C (30v+blk) | Dijkstra | 86.11 | 0.105 | 100% | 0 | 20.4 |
|  | Static A* | 90.56 | 0.105 | 100% | 0 | 20.6 |
|  | DAA | 86.11 | 0.105 | 100% | 5 | 15.2 |
| D (50v) | Dijkstra | 80.33 | 0.099 | 100% | 0 | 21.0 |
|  | Static A* | 83.00 | 0.099 | 100% | 0 | 21.2 |
|  | DAA | 80.33 | 0.099 | 100% | 7 | 15.4 |

## Result Graphs

Fig. 2 compares energy consumption across all scenarios and algorithms. DAA consistently saves over 20 % by combining smoother IDM trajectories with proactive avoidance of stop-and-go zones.

Energy consumption (kWh/100 km) per algorithm per scenario. DAA achieves 21.8–26.7 % savings over baseline algorithms.

Fig. 3 shows travel time. The gap between static A* and DAA is most visible in Scenario C (blockage), where DAA’s proactive bypass avoids the 4.45 s penalty that static A* incurs by committing to the blocked corridor.

Mean travel time (s) per algorithm per scenario. DAA matches Dijkstra in all scenarios and avoids the 4.45 s penalty of static A* in the blockage scenario (C).

Fig. 4 shows DAA replanning activity. Replan count grows with fleet size and hazard intensity, confirming that the router actively responds to dynamic conditions rather than committing to a fixed path.

Number of DAA replanning events per scenario. The spike in Scenario C corresponds to the roundabout blockage event triggering multiple bypass re-optimisations.

# Discussion

The energy savings are real and consistent across all four scenarios. They come from the IDM subsystem’s smooth acceleration profiles as much as from better routing—in Scenarios A and B, where all three algorithms choose the same path, DAA still saves roughly 22 % simply by eliminating unnecessary braking cycles. This is worth being transparent about: the routing contribution and the trajectory-execution contribution are coupled in these tests, and disentangling them would require holding the trajectory model constant across algorithms.

Scenario C offers the cleaner demonstration of the routing benefit. Static A* commits to the main corridor and accumulates a 4.45-second penalty; DAA, forewarned by the DENM alert, re-routes five times and matches Dijkstra’s time while additionally keeping the ego vehicle out of the hazard zone entirely. That qualitative difference—avoiding the accident field rather than just being delayed by it—does not appear in the travel time column, which is a limitation of the metrics we chose.

We also want to be direct about the 100 % classifier accuracy. The labels are deterministic functions of the features through a hard kinematic threshold, so the model is learning a physical law rather than a statistical pattern. It will not necessarily generalise to conditions outside the NGSIM distribution—fog, mixed AV-and-human traffic, or two-wheelers—and we have not tested it on such conditions.

## Limitations and Future Work

The road graph we used has seven nodes and two corridors. Real urban networks have hundreds of intersections and introduce turn priorities, pedestrian crossings, and merging conflicts that our routing layer does not currently handle. The NGSIM dataset represents a single highway in a single city, so geographic and cultural generalisation is untested. The simulation uses a custom JavaScript engine rather than a validated physics tool like CARLA or SUMO, limiting the fidelity of the vehicle dynamics. Future work will address CARLA-based validation, extension to full urban intersection graphs, and an online continual-learning component that updates the risk model from live fleet telemetry.

# Conclusion

The central argument of this paper is that routing and safety prediction do not have to be separate concerns. By training two Random Forest models on a million-frame trajectory dataset and embedding their predictions directly into an A* cost function, we built a router that responds to risk before the vehicle encounters it rather than after. The accompanying subsystems—IDM trajectories, V2X alerts, EKF localisation, VT-Micro energy tracking, and a fault-tolerant inference wrapper—turn the routing algorithm into a full navigation stack testable end-to-end. Results show 100 % route completion across fleet sizes of 10–50 vehicles, 21.8 % mean energy savings, and verified robustness to six categories of sensor fault.

All code, trained models, and the interactive simulation cockpit are at https://github.com/reddyakshith2502-dotcom/AUTOPLAN.

99

D. González, J. Pérez, V. Milanés, and F. Nashashibi, “A review of motion planning techniques for automated vehicles,” IEEE Trans. Intell. Transp. Syst., vol. 17, no. 4, pp. 1135–1145, Apr. 2016.

E. W. Dijkstra, “A note on two problems in connexion with graphs,” Numer. Math., vol. 1, no. 1, pp. 269–271, 1959.

P. E. Hart, N. J. Nilsson, and B. Raphael, “A formal basis for the heuristic determination of minimum cost paths,” IEEE Trans. Syst. Sci. Cybern., vol. 4, no. 2, pp. 100–107, Jul. 1968.

S. Koenig and M. Likhachev, “D* Lite,” in Proc. AAAI Conf. Artif. Intell., Edmonton, Canada, 2002, pp. 476–483.

S. Shalev-Shwartz, S. Shammah, and A. Shashua, “Safe, multi-agent, reinforcement learning for autonomous driving,” arXiv:1610.03295, 2016.

F. Altché and A. de La Fortelle, “An LSTM network for highway trajectory prediction,” in Proc. IEEE ITSC, Yokohama, Japan, 2017, pp. 353–359.

L. Breiman, “Random forests,” Mach. Learn., vol. 45, no. 1, pp. 5–32, 2001.

J. C. Hayward, “Near-miss determination through use of a scale of danger,” Highw. Res. Rec., no. 384, pp. 24–34, 1972.

B. L. Allen, B. T. Shin, and P. J. Cooper, “Analysis of traffic conflicts and collisions,” Transp. Res. Rec., no. 667, pp. 67–74, 1978.

Federal Highway Administration, “NGSIM US Highway 101 Dataset,” U.S. Dept. of Transportation, 2006. [Online]. Available: https://ops.fhwa.dot.gov/trafficanalysistools/ngsim.htm

ETSI, “Intelligent Transport Systems; Cooperative Awareness Basic Service,” ETSI EN 302 637-2, v1.4.1, 2019.

B. Schünemann, “V2X simulation runtime infrastructure VSimRTI,” Comput. Netw., vol. 55, no. 14, pp. 3189–3198, 2011.

M. Treiber, A. Hennecke, and D. Helbing, “Congested traffic states in empirical observations and microscopic simulations,” Phys. Rev. E, vol. 62, no. 2, pp. 1805–1824, 2000.

H. Rakha, K. Ahn, and A. Trani, “Development of VT-Micro model for estimating hot stabilized light duty vehicle and truck emissions,” Transp. Res. Part D, vol. 9, no. 1, pp. 49–74, 2004.

O. Khatib, “Real-time obstacle avoidance for manipulators and mobile robots,” Int. J. Robot. Res., vol. 5, no. 1, pp. 90–98, 1986.

Source code and simulation: https://github.com/reddyakshith2502-dotcom/AUTOPLAN